| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | ATOMIC FORCE MICROSCOPE | Description Modular microscope and electronics Integrated stage control Built-in access to signal routing Features Closed-Loop Accuracy Large Sample Stage Automatic Image Optimization Technical specification X-Y Scan Range 90μm x 90μm typical, 85μm minimum Z Range 10μm typical in imaging and force ramp modes, 9.5μm minimum Vertical Noise Floor <50pm RMS in appropriate environment, typical imaging bandwidth (up to 625Hz) XY Position Sensor Noise Level (Closed Loop) <0.5nm RMS typical imaging bandwidth (up to 625Hz) Z Position Sensor Noise Level (Closed Loop) <0.2nm RMS typical imaging bandwidth (up to 625Hz) Sample/Size/Holder 150mm vacuum chuck, 15mm thick; Up to 40mm thick with optional frame spacer Motorized Positioning Stage (X-Y axis) 150mm x 150mm inspectable area; Programmable for multi-site measurements Microscope Optics 5-megapixel digital camera;180μm to 1465μm viewing area; Digital zoom and motorized focus Signal Access Configurable I/O signal access built into controller; Includes customizable signal routing, digital feedback, and dual digital lock-in Single Point Spectroscopy Three-axis closed loop control for point-and-shoot positioning and ramping; Spring constant calibration with built-in thermal tune Sample Temperature Control -35 to +250°C with optional heater/cooler accessory; Includes gas purging capability Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |