| Item No. | Item Name | Technical Specifications | Unit of Measurement | Quantity |
| --- | --- | --- | --- | --- |
| 1 | ATOMIC FORCE MICROSCOPE | Technical specification X-Y Scan Range (μm) ≥90 x 90 Z Range (μm) ≥12 Vertical Noise Floor- RMS/Adev (pm) :≤30/25, height in appropriate environment, typical imaging BW (≤625 Hz) X-Y Position Noise - RMS/Adev, Closed-Loop (pm) ≤125/100 Z Sensor Noise- RMS/Adev (pm) ≤20/15 typical imaging BW (≤625 Hz) System Drift <200 pm/min Sample Size / Holder 210 mm vacuum chuck for samples ≤210 mm in diameter, ≤15 mm thick Motorized Position 150 × 180 mm inspectable area with manually rotating chuck; 2 μm repeatability, unidirectional; 3 μm repeatability, bidirectional Optics Auto focus and digital zoom; Documentation: - CE marking and declaration conformity - Documents for the Installation and Operational Qualification (IQ / OQ) - Functional Specification - Design Qualification (DQ) and others relevant document | Pcs | 1 |